MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

1

Multi-Horizon Hazard Models for Battery Failure

Prediction:

Within-Dataset Reliability,

Cross-Chemistry Transferability,

and Embedded

Edge Deployment

Anonymous

horizon H, the model predicts whether failure, defined as

Abstract—Predicting whether

a lithium-ion cell

will

fail

inside a short operating window is a different question from

SOH below 0.80 or average voltage sag beyond 6%, occurs

estimating its

remaining useful

life,

and it

is

the one that

within the next H cycles. Their histogram-based gradient

matters most for real-time dispatch in electric vehicles and grid

boosting model

on the NASA 18650 set (37 LCO cells)

storage.

This paper widens a recently proposed multi-horizon

reached AUC of 0.87–0.90 across H ∈ {10, 20, 30, 50}.

hazard classification framework, first shown on one model and

That work left three gaps open.

First,

one model

on

one dataset, into a study that covers four model families, three

LCO training configurations,

and two lithium-iron-phosphate

one dataset says nothing about sensitivity to the learner,

(LFP) transfer targets.

Within-dataset reliability holds across

the hyperparameters,

or the calibration method.

Second,

the board, with area under the curve (AUC) of 0.85 or better

there was no cross-chemistry check,

so nothing stopped

in 30 of 32 configurations under Platt calibration.

A fairness-

a model

trained on LCO from being trusted on LFP

corrected comparison shows

Platt

scaling beats

isotonic re-

or

NMC cells.

Third,

no path existed from a Python

gression on discrimination,

with the largest gap on the long-

tailed CALCE set (AUC 0.713 to 0.887).

The central

result

research prototype to the microcontroller where a battery

is a controlled State-of-Health (SOH) ablation: removing SOH

management system actually runs.

collapses cross-chemistry AUC from 0.76–1.00 to 0.32–0.61 on

This paper closes all

three.

The contributions are:

(i)

Oxford LFP and to 0.54–0.88 on the 141-cell

MIT–Stanford

a four-model benchmark, three tree ensembles (XGBoost,

−64

Severson set, with DeLong p values ranging down to 2.6×10

−100

LightGBM,

Random Forest) plus a GRU sequence clas-

on Oxford and below 10

on Severson.

SOH therefore acts

as a chemistry-specific lookup key rather than a transferable

sifier,

run on two LCO datasets (NASA 18650,

CALCE

degradation signal,

and isotonic calibration fails

to transfer

LCO/CX2) under a unified 5-fold GroupKFold protocol;

as well. On the deployment side, all three tree ensembles (900

(ii)

a controlled LCO-to-LFP transfer

study onto two

trees, 26,336 nodes) pack into a 372 kB binary and run on a $12

independent targets,

the 5-cell

Oxford pouch set and the

ESP32-S3 through a hand-written C tree walker,

reproducing

−6

141-cell

MIT–Stanford Severson set,

with and without

the Python training libraries to within 1.1 × 10

across all

1,028 validation rows,

with measured single-row latency of

SOH as

a feature,

analyzed with DeLong significance

0.50–1.29 ms with all

three models in PSRAM and 0.32–0.50

tests

and SHAP explanations;

(iii)

a fairness-corrected

ms for a single model

held in internal

SRAM.

comparison of

isotonic

and Platt

calibration;

and (iv)

Index Terms—Battery failure prediction, multi-horizon haz-

a complete embedded deployment

that

packs

all

three

ard classification, cross-chemistry transfer, probability calibra-

ensembles into one flat binary and runs them on an ESP32-

tion,

SHAP,

embedded machine learning,

ESP32-S3.

S3 through a hand-written C tree walker.

Removing SOH drops cross-chemistry AUC from near-

I.

Introduction

perfect to near-chance, and the drop comes from a specific

L

ITHIUM-ION batteries sit underneath the electrifi-

mechanism rather than random noise.

Trees exploit SOH

cation of transport and the decarbonization of grids,

with isolated hard splits

that

behave like a chemistry-

yet

their

long-term reliability remains

hard to certify

specific capacity-to-horizon lookup table,

and that table

in the field.

Conventional

prognostics casts the problem

does not transfer.

A second,

independent result is that

as remaining-useful-life (RUL) regression,

where a model

calibration methods

themselves

fail

to

transfer,

since

predicts a continuous cycle count to end of life [2]–[4]. RUL

isotonic

regression bins

distribution-shifted scores

into

regression is fine for capacity-fade studies,

but it answers

degenerate steps.

a question operators rarely ask in real

time.

A battery

management system needs to know whether a specific cell

II.

Related Work

will survive the next mission, not how many cycles it has

A.

From RUL regression to hazard classification

left in aggregate.

Shikdar and Laaksonen [1] recast the problem as multi-

Data-driven battery prognostics

has

historically been

horizon hazard classification.

For

a given cycle

t

and

regression.

Meng and Li

[2]

group prior

methods

into

physics-based,

purely data-driven,

and hybrid families.

This manuscript is under review;

author identities withheld for

Tree ensembles, such as Random Forest [3], XGBoost [4],

blind review.

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

2

and LightGBM [5], have been applied to capacity and RUL

discharge cycle t in cell c is labeled as failing within horizon

estimation,

but their use for binary hazard classification

H if any cycle k ∈ [t, t+H) meets either of two criteria,



at multiple horizons is recent [1].

(c)



1,

∃ k ∈ [t, t+H) :

SOH

≤ 0.80 or





k

(c,H)

¯

¯

(c)

(2)

y

=

(c)

V

< 0.94

V

,

B.

Cross-chemistry and transfer learning

t



k





0,

otherwise,

Transfer learning for battery state estimation is picking

up.

Sahoo et al.

[10]

fine-tuned an SOH estimator across

and once triggered the label stays 1 for all later cycles. The

NASA and CALCE cells,

and Lu et al.

[11]

pretrained

task is to estimate the conditional

probability of

failure

on large-scale degradation data before adapting to new

over the horizon,

cell

types.

Both target regression-based SOH estimation

(

)

(c,H)

(c)

rather

than classification-based hazard prediction.

The

p

= Pr

T

≤ t+H | T

> t,

x

,

(3)

f

f

t

t

hazard setting adds two complications: the label depends

(c)

where T

is the failure cycle and x

is the feature vector

on the chosen horizon H, and the calibration of probability

f

t

measured at cycle t.

Datasets without voltage data,

or

outputs,

which decisions depend on,

may degrade under

where voltage sits at the cutoff as in Oxford’s flat LFP

distribution shift.

plateau,

fall

back to the SOH criterion alone.

C.

Calibration of classifier outputs

B.

Datasets

Post-hoc calibration maps raw classifier scores to prob-

Four public aging datasets are used.

NASA 18650 [6]

abilities.

Platt scaling [12]

fits a sigmoid through logistic

has 37 LCO cells aged under randomized charge–discharge

regression on the model’s scores, while isotonic regression

profiles at room temperature, roughly 1,000 cycles per cell,

[13]

fits a non-decreasing step function.

Niculescu-Mizil

with diverse failure modes. CALCE LCO/CX2 [7] provides

and Caruana [14]

showed Platt is safer on small

datasets

7 cells (CS2_33–36, CX2_36–38) at 1C/1C with per-cycle

and isotonic can overfit,

and Huang et al.

[15]

extended

voltage, current, and capacity. On the LFP side, Oxford [8]

this to imbalanced classification.

No prior study,

to our

has 5 pouch cells at 1C/1C under controlled temperature

knowledge, has compared the two calibrators under cross-

with 300–500 cycles each, and MIT–Stanford Severson [9]

chemistry distribution shift in battery prognostics.

has

141 LFP cells

under

a fast-charging protocol

with

534–2,237 cycles each (about 117K total).

D.

Explainability and edge deployment

TreeSHAP [16]

gives

consistent

feature

attributions

C.

Features and preprocessing

grounded in Shapley values and has become the standard

Each cycle is

described by seven scalar

features:

cy-

tool

for

diagnosing

which inputs

drive

an ensemble.

cle index,

average discharge voltage,

minimum discharge

On deployment,

moving inference onto a microcontroller

voltage,

average discharge current,

average temperature,

removes latency,

connectivity requirements,

and privacy

discharge duration, and SOH. Trees are scale-invariant, so

concerns [17].

Porting tree ensembles to embedded plat-

no standardization is applied to them;

the GRU receives

forms through model-to-code compilation has been done

per-feature standardization using training-set

statistics.

before [18],

but the specific combination of multi-horizon

Missing CALCE values

(temperature and duration are

hazard models with a hand-written C tree walker on a

unlogged) are filled with zero.

$12 ESP32-S3 has not.

III.

Methodology

D.

Tree models and hyperparameters

A.

Composite failure label

Three tree ensembles are benchmarked with hyperpa-

rameters matched to the original

study [1]:

XGBoost [4]

Following the original

protocol

[1],

the State-of-Health

with max_depth 4, 300 estimators, learning rate 0.05, sub-

of

cell

c at cycle k is the ratio of

the discharge capacity

sample 0.8, colsample_bytree 0.8, and min_child_weight

to the mean capacity over the first 10 cycles,

5; LightGBM [5] with the same depth, count, and rate; and

(c)

Random Forest [3] with max_depth 6 and 300 estimators.

Q

(c)

k

,

SOH

=

All

use random_state 42 for deterministic replication.

¯

k

Q

(c)

(1)

∑

10

1

¯

(c)

(c)

Q

,

Q

=

E.

GRU sequence classifier

i

10

i=1

A compact

GRU tests

whether

sequence-aware

rep-

(c)

where

Q

is

the

discharge

capacity at

cycle

k.

The

resentations

capture

trajectories

that

transfer

across

k

¯

(c)

early-life voltage baseline is defined analogously,

V

=

chemistries.

It ingests a sliding window of W =10 consec-

∑

¯

¯

(c)

(c)

10

1

V

,

with

V

the average discharge voltage.

A

utive cycles per cell as a 10×f tensor and runs one GRU

i

i

i=1

10

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

3

layer with 8 hidden units.

At each timestep the standard

since pooling conflates within-cell

ranking with between-

update equations apply,

cell differences and can hide degenerate behavior where a

model

assigns nearly identical

scores to every cell.

r

= σ(W

x

+ U

h

),

t

r

t

r

t−1

z

= σ(W

x

+ U

h

),

t

z

t

z

t−1

H.

Evaluation and statistical

testing

(4)

˜

h

= tanh(W

x

+ U

(r

⊙ h

)),

Within-dataset

evaluation applies

5-fold GroupKFold

t

h

t

h

t

t−1

with cells

as

grouping

units,

so

every

cycle

from a

˜

h

= (1 − z

) ⊙ h

+ z

⊙

h

,

t

t

t−1

t

t

given cell

stays in one fold.

This removes the optimistic

and the final

hidden state maps through a linear layer

bias from leaking a cell’s cycles into both training and

⊤

with sigmoid activation,

p = σ(w

h

+ b),

to give the

test.

Models are retrained from scratch per horizon.

The

W

failure probability.

Adam with learning rate 0.005 and

DeLong nonparametric test [19]

for paired ROC curves

binary cross-entropy loss are used.

The 8-unit width was

judges whether the AUC difference between with-SOH and

chosen to match the small

number of training cells (7 to

without-SOH conditions is significant.

The test evaluates

37); wider layers overfit severely. For cross-chemistry runs

the normalized difference of the two areas,

the feature set is reduced to avoid unit and availability

A

− A

1

2

mismatches:

average

current

is

dropped (amperes

for

√

,

(7)

z =

V

+ V

− 2V

LCO,

milliamperes

for

LFP),

as

are

temperature

and

11

22

12

duration.

The remaining features,

cycle number,

average

where A

is the empirical area under the j-th ROC curve

j

voltage,

minimum voltage,

and SOH when enabled,

are

and V

are the elements of its covariance matrix estimated

ij

standardized with training-set statistics.

nonparametrically from the same test instances.

F.

Calibration protocol

I.

Embedded deployment architecture

Two post-hoc calibration schemes are compared under

The

deployment

pipeline

exports

the

three

trained

a fairness-corrected protocol.

Platt scaling fits a sigmoid

models into one flat binary and runs them on an ESP32-

to the base model’s raw scores s

,

S3 through a hand-written C tree

walker.

The

layout

i

is deliberately simple:

a 12-byte file header,

a 3×4-byte

1

,

(5)

p

=

offset table,

and per-model

sections holding a 4-byte tree

i

1 + exp(−αs

− β)

i

count followed by consecutive per-tree node arrays.

Each

by

high-regularization

logistic

regression

node stores the split feature index,

the split threshold as

10

(LogisticRegression,

C=10

).

Isotonic

regression

fits

a 32-bit

float,

and child indices;

leaves store the class-

the non-decreasing step function that minimizes squared

1 probability directly.

The walker

is one C function of

error,

∑

(

)

about 70 lines that follows child pointers,

sums the leaf

2

g =

arg min

y

− g(s

)

,

(6)

contributions per tree,

and applies the per-model

output

i

i

g non-decreasing

i

transform.

Writing ℓ

for the leaf

value reached in tree t

t

implemented with scikit-learn’s

IsotonicRegression and

and T for the number of

trees,

Random Forest averages

out-of-bounds clipping. Both calibrators are fit on the base

and clamps its leaf probabilities,

model’s training-fold scores, so they share identical inputs

)

(

∑

T

1

and any difference in output comes from the calibration

ℓ

,

0,

1

,

(8)

p = clamp

t

function alone, isolating it from the confound of retraining

T

t=1

or data splitting.

while XGBoost and LightGBM pass the summed score

A reproducibility note is warranted. The original study

through a sigmoid,

[1]

reported Brier scores around 0.032;

our re-execution

1

of

the released code yields Brier values between roughly

,

(9)

p =

(

)

∑

0.09 and 0.29,

an eightfold-or-larger

gap we could not

T

1 + exp

−

ℓ

− s

t

0

t=1

resolve from the available source. Our AUC values (0.80–

where s

is the model’s initialization score (s

̸

= 0 only

0

0

0.90) fall

within the published range,

and our qualitative

for XGBoost).

Fig.

1 shows the wiring.

Three validation

conclusions hold. The Brier gap does not affect the relative

stages confirm correctness:

a Python walker that mirrors

comparisons that form the core of this paper.

the C logic and is compared against predict_proba(),

a

cross-compiled C binary tested on desktop x86_64,

and

G.

Cross-chemistry transfer protocol

on-target execution on an ESP32-S3-DevKitC-1 reading

Transfer is evaluated by training on LCO cells (NASA

the binary from flash.

All

three must report maximum

alone,

CALCE alone,

or both) and testing independently

−5

absolute error below 10

on every row before deployment

on each cell of the two LFP targets. Three configurations

is accepted.

are explored:

NASA→LFP (37 cells),

CALCE→LFP (7

cells),

and ALL-LCO→LFP (44 cells).

Each is evaluated

with and without SOH. Performance is reported as per-cell

mean ± standard deviation rather than one pooled score,

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

4

Fig.

1.

Prototype wiring for the ESP32-S3 deployment.

The battery discharges through the INA219 shunt into a load,

the 2:1 divider

senses cell voltage at ADC GPIO1, and I2C (GPIO8/9) serves the INA219 and DS3231 while OneWire (GPIO10) reads the DS18B20. The

boards are a $12 ESP32-S3-DevKitC-1 and two breakout modules.

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

5

IV.

Results

A.

Within-dataset performance

Fig. 2.

Within-dataset AUC heatmap, mean across horizons, Platt-

calibrated.

Table I summarizes within-dataset performance across

the

four

horizons

for

both LCO datasets

under

Platt

calibration.

Discrimination is

strong throughout.

Mean

AUC runs from 0.844 (GRU on CALCE) to 0.913 (Ran-

dom Forest on NASA and XGBoost on CALCE).

Every

tree-model

configuration clears

the

0.85 bar;

the

only

two configurations

that

fall

short

are

both the

GRU

on CALCE,

at H=20 (0.779) and H=30 (0.768),

where

the

sequence

model

overfits

a

dataset

whose

per-cell

degradation tails run to nearly 2,000 cycles.

B.

Platt versus isotonic calibration

Fig. 3.

Reliability diagrams for NASA (top) and CALCE (bottom),

XGBoost at H=20. Isotonic produces degenerate bins on long-tailed

CALCE data.

C.

Cross-chemistry transfer with and without SOH

Table II compares the two calibration methods, (5) and

Table III reports transfer at H=20 with SOH kept. Tree

(6), averaged over models and horizons. Brier scores barely

models reach high raw AUC across every configuration,

move (NASA:

0.209 isotonic versus 0.212 Platt;

CALCE:

while the GRU struggles:

its distributed hidden state en-

0.107 versus

0.105).

The AUC picture is

different.

On

tangles SOH with chemistry-specific features under shift.

NASA,

Platt

lifts

mean AUC from 0.838 to 0.903;

on

Removing SOH changes the picture completely (Table IV).

CALCE the

gain is

much larger,

0.713 to 0.887.

The

On Oxford,

raw AUC falls

to 0.32–0.61 across

all

tree

asymmetry comes from CALCE’s long-tailed degradation

models and all training configurations, at or below chance.

(up to

1,952

cycles

per

cell),

which pushes

extreme

On Severson the drop is smaller in absolute terms (0.54–

scores into degenerate isotonic bins, while Platt’s sigmoid

0.88)

but

the

within-pair

gap stays

around 0.14–0.16

compresses them smoothly.

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

6

TABLE I

Within-Dataset Platt-Calibrated AUC and Brier Scores Across Four Horizons. GroupKFold by Cell; Values Are Fold Means.

Model

Dataset

H=10

H=20

H=30

H=50

Mean AUC

Brier

XGBoost

NASA

0.903

0.899

0.910

0.918

0.907

0.197

XGBoost

CALCE

0.916

0.926

0.911

0.899

0.913

0.101

LightGBM

NASA

0.912

0.909

0.892

0.901

0.903

0.250

LightGBM

CALCE

0.908

0.924

0.886

0.916

0.909

0.101

Random Forest

NASA

0.911

0.901

0.912

0.927

0.913

0.209

Random Forest

CALCE

0.895

0.889

0.884

0.864

0.883

0.095

GRU

NASA

0.930

0.884

0.876

0.868

0.889

0.191

GRU

CALCE

0.944

0.779

0.768

0.885

0.844

0.124

TABLE II

Calibration Method Comparison Averaged Over Models and

Horizons. Both Methods Operate on Identical Training-Fold Scores.

Dataset

Iso. AUC

Platt AUC

Iso. Brier

Platt Brier

NASA

0.838

0.903

0.209

0.212

CALCE

0.713

0.887

0.107

0.105

Fig.

5.

Cross-chemistry transfer

without

SOH,

raw AUC.

Top:

Oxford, bottom: Severson.

D.

GRU entanglement under distribution shift

The GRU shows

an architecture-specific failure that

differs

from the

trees.

Its

8-dimensional

hidden state

compresses SOH together with voltage, current, and cycle

trends

across

the 10-step window.

Under

LCO-to-LFP

shift the entangled voltage and cycle components partially

Fig. 4.

Cross-chemistry transfer with SOH, raw AUC. Top: Oxford,

bottom: Severson.

corrupt the SOH signal,

producing raw AUC as low as

0.077 for CALCE-to-Oxford at H=20 against 0.76–0.84

for

the trees.

A separate mechanism drives

a CALCE-

trained GRU to invert its Oxford rankings (AUC about

AUC,

and the DeLong test

(Section IV-F)

confirms

it

0.03–0.12): CALCE’s 92% composite-failure rate saturates

is significant.

the learned decision boundary, which then ranks Oxford’s

The combined evidence points to SOH as a chemistry-

feature distribution backwards.

specific lookup key rather

than a transferable feature.

With SOH available,

a model

learns

a mapping such

E.

Calibration methods themselves fail

to transfer

as “SOH 0.85 means roughly 50 cycles to failure” from

LCO data and applies it unchanged to LFP,

where the

A second,

independent

failure

appears

in the

cross-

mapping is numerically similar for unrelated electrochem-

chemistry tables

when isotonic

calibration is

applied.

ical

reasons.

Without SOH,

the remaining features carry

Tree

models

lose

0.15–0.48 AUC points

depending on

no chemistry-invariant signal suﬀicient for cross-chemistry

the configuration. The collapse happens because isotonic’s

discrimination.

step function is fit to the training distribution; when test

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

7

TABLE III

Cross-Chemistry Transfer at H=20 With SOH. AUC Reported as Per-Cell Mean Across Test Cells.

Training

Model

Target

Raw AUC

Iso. AUC

NASA

XGBoost

Oxford

0.959

0.773

NASA

XGBoost

Severson

0.997

0.532

NASA

LightGBM

Oxford

1.000

0.969

NASA

LightGBM

Severson

0.992

0.988

NASA

Random Forest

Oxford

1.000

0.782

NASA

Random Forest

Severson

0.999

0.523

CALCE

XGBoost

Oxford

0.763

0.510

CALCE

XGBoost

Severson

0.957

0.624

CALCE

LightGBM

Oxford

0.837

0.513

CALCE

LightGBM

Severson

0.952

0.617

CALCE

Random Forest

Oxford

0.766

0.510

CALCE

Random Forest

Severson

0.996

0.664

ALL LCO

XGBoost

Oxford

0.826

0.510

ALL LCO

XGBoost

Severson

0.995

0.630

ALL LCO

LightGBM

Oxford

0.969

0.510

ALL LCO

LightGBM

Severson

0.993

0.580

ALL LCO

Random Forest

Oxford

0.992

0.572

ALL LCO

Random Forest

Severson

0.996

0.952

TABLE IV

Cross-Chemistry Transfer at H=20 Without SOH.

Training

Model

Target

Raw AUC

Iso. AUC

NASA

XGBoost

Oxford

0.519

0.508

NASA

XGBoost

Severson

0.676

0.516

NASA

LightGBM

Oxford

0.448

0.448

NASA

LightGBM

Severson

0.535

0.491

NASA

Random Forest

Oxford

0.442

0.541

NASA

Random Forest

Severson

0.836

0.500

CALCE

XGBoost

Oxford

0.360

0.513

CALCE

XGBoost

Severson

0.806

0.614

CALCE

LightGBM

Oxford

0.410

0.513

CALCE

LightGBM

Severson

0.830

0.606

CALCE

Random Forest

Oxford

0.435

0.510

CALCE

Random Forest

Severson

0.870

0.622

ALL LCO

XGBoost

Oxford

0.415

0.510

ALL LCO

XGBoost

Severson

0.860

0.725

ALL LCO

LightGBM

Oxford

0.316

0.510

ALL LCO

LightGBM

Severson

0.808

0.595

ALL LCO

Random Forest

Oxford

0.608

0.541

ALL LCO

Random Forest

Severson

0.876

0.836

scores shift, several different scores land in one bin and get

TABLE V

DeLong Test p-Values for the SOH Ablation (ALL-LCO → LFP,

identical calibrated probabilities, which drags AUC down

H=20). AUCs Are Pooled Across Test Cells (See Table III for

through tied predictions.

Per-Cell Means). All Comparisons Are Significant at α = 0.05.

F.

DeLong test:

statistical

significance of

the SOH abla-

Target

Model

AUC w/ SOH

AUC w/o SOH

p-value

tion

−51

Oxford

XGBoost

0.917

0.429

7.2 × 10

−64

Oxford

LightGBM

0.971

0.332

2.6 × 10

Table V reports DeLong p-values,

computed with (7),

−36

Oxford

Random Forest

0.989

0.581

6.5 × 10

−100

Severson

XGBoost

0.896

0.750

< 10

for the ALL-LCO→LFP ablation at H=20. On Oxford the

−100

Severson

LightGBM

0.882

0.718

< 10

−36

values range from 6.5 × 10

(Random Forest) down to

−100

Severson

Random Forest

0.889

0.746

< 10

−64

2.6 × 10

(LightGBM); on Severson all three fall below

−100

10

. The test accounts for the correlation between with-

SOH and without-SOH AUCs

computed on the

same

SOH kept: SOH dominates by a wide margin, cycle index

instances,

so these numbers

are decisive rather

than a

a distant second,

and the voltage,

current,

temperature,

quirk of paired sampling.

and duration features contribute little.

Figs.

9–11 repeat

the same comparisons with SOH removed,

and every re-

G.

SHAP feature importance

maining feature collapses to near-zero SHAP spread. That

visual collapse matches the quantitative AUC collapse and

TreeSHAP [16]

gives a direct test of

the lookup-table

reads as a one-glance demonstration of the paper’s central

hypothesis.

Figs.

6–8 show the attributions for the three

negative result.

tree models in NASA-to-Oxford transfer at H=20 with

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

8

TABLE VI

Embedded Deployment: Model Size, Accuracy, On-Device Validation Error, and Measured Inference Time. Errors Are Maximum |C

engine − predict_proba()| Over All 1,028 Rows, Measured on the ESP32-S3. Timings Measured On-Device at 240 MHz With the Binary

in Flash (PROGMEM), PSRAM, or a Single Model in Internal SRAM. The AUC Column Is the 5-Fold GroupKFold Platt Value at H=20

From Table I (Deployed Models Are Retrained on the Full Data).

Model

Nodes

AUC (H=20)

Max Err

Flash

PSRAM

SRAM (1 model)

−7

XGBoost

3,272

0.899

2.4 × 10

0.76 ms

0.50 ms

0.32 ms

−6

LightGBM

7,000

0.909

1.1 × 10

1.55 ms

0.93 ms

0.43 ms

−8

Random Forest

16,064

0.901

3.0 × 10

2.36 ms

1.29 ms

0.50 ms

All three

26,336

—

—

4.7 ms

2.7 ms

—

Fig. 6.

SHAP feature importance for XGBoost in NASA-to-Oxford

Fig. 7.

SHAP feature importance for LightGBM in NASA-to-Oxford

transfer at H=20, with SOH. SOH dominates by a wide margin.

transfer at H=20, with SOH. SOH dominates by a wide margin.

SRAM. The full 372 kB binary does not fit internal SRAM,

H.

Embedded deployment validation

whose free heap is roughly 320 kB,

but each individual

Three independent validation stages confirm that the

model

does (XGBoost is about 46 kB,

LightGBM 98 kB,

C tree engine reproduces

the Python training libraries

Random Forest 225 kB), and the firmware runs one model

through the output transforms of

(8) and (9):

a Python

at a time, so the SRAM-resident numbers are the realistic

walker over the extracted trees,

the same walker cross-

per-inference cost. Both flash and PSRAM land well above

checked on the

full

1,028-row set,

and the

C engine

the 150–250 µs we first projected, because the working set

run

on-device

on

an

ESP32-S3

against

the

library’s

exceeds the ESP32-S3’s data cache and nearly every node

predict_proba().

Fig.

13 plots

every one

of

the

1,028

read misses;

PSRAM is about 1.5× faster than flash for

on-device predictions

against

the Python values

for

all

the same reason.

The all-three PSRAM path still

clears

three models;

the points sit on the identity line and the

the 100 ms per-cycle budget by roughly 37×, and a single

residual histograms are pinned to zero. The measured on-

model

from SRAM runs close to the original

projection.

−7

device maximum absolute error is 2.4×10

for XGBoost,

−6

−8

1.1 × 10

for LightGBM,

and 3.0 × 10

for Random

V.

Discussion

−5

Forest,

all

far inside the 10

tolerance.

A.

The SOH-as-lookup-table mechanism

Table VI lists the models’ sizes, their NASA H=20 Platt

AUC,

the on-device validation error,

and the measured

The

central

negative

result

reads

best

as

a leakage

inference time,

and Fig.

12 plots the latency,

the speed–

diagnosis rather than a generic failure of transfer learning.

accuracy trade-off, and the footprint. The three ensembles

SOH is a capacity ratio, and capacity declines with age in

occupy 372 kB in the flat

binary.

Measured single-row

both LCO and LFP cells, so both chemistries traverse SOH

inference at 240 MHz depends on where the binary sits:

values from 1.0 down to 0.8. What differs is the mapping

0.76 ms,

1.55 ms,

and 2.36 ms for XGBoost,

LightGBM,

from SOH to remaining cycles:

LCO degrades faster per

and Random Forest from memory-mapped flash; 0.50 ms,

cycle,

so a given SOH corresponds

to fewer

remaining

0.93 ms,

and 1.29 ms from PSRAM;

and 0.32 ms,

0.43

cycles.

A model

that learned the LCO mapping assigns

ms, and 0.50 ms when a single model is placed in internal

high failure

probability at

the

appropriate

LCO cycle

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

9

Fig.

8.

SHAP feature importance for Random Forest in NASA-

Fig.

10.

SHAP feature importance for

LightGBM in NASA-to-

to-Oxford transfer at H=20,

with SOH.

SOH dominates by a wide

Oxford transfer at H=20, without SOH. All features collapse to near-

margin.

zero spread.

Fig.

11.

SHAP feature importance for Random Forest in NASA-

Fig. 9.

SHAP feature importance for XGBoost in NASA-to-Oxford

to-Oxford transfer at H=20,

without SOH.

All

features collapse to

transfer at H=20,

without SOH.

All

features collapse to near-zero

near-zero spread.

spread, so no chemistry-invariant signal remains.

on a $12 microcontroller. Second, cross-chemistry transfer

count even on LFP test cells,

which produces high AUC,

should not

be

assumed;

without

SOH no model

class

but

the

prediction rides

on a chemistry-specific

proxy

clears

chance,

and with SOH the

GRU’s

distributed

rather than any real

understanding of

LFP degradation.

representation still

offers no guarantee of

generalization.

SHAP confirms

that

SOH carries

the

dominant

share

Third, post-hoc calibration, especially isotonic regression,

of

feature importance in every model,

and removing it

should not be applied blindly under distribution shift; raw

collapses every predictor to near-random performance.

scores give a more dependable discriminative signal

for

cross-chemistry deployment.

B.

Implications for battery management practitioners

Three practical

points

follow.

First,

within-chemistry

C.

Limitations

hazard models built from standard charge and discharge

features (voltage,

current,

temperature,

cycle count) are

The

Oxford set

holds

only five

cells,

too few for

a

reliable,

reaching AUC of

0.85 and above

with Platt

reliable within-dataset evaluation and enough to blunt the

calibration,

and the deployment

results

show they run

precision of

the per-cell

standard deviation;

the 141-cell

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

10

gets and a complete embedded deployment on an ESP32-

S3. Three findings stand out. Within-dataset performance

is consistent across model

classes (mean AUC 0.84–0.91)

and Platt

calibration beats

isotonic throughout.

Cross-

chemistry transfer fails for every model

class once SOH

is removed,

and the failure comes from SOH acting as

a chemistry-specific lookup key rather

than a transfer-

able degradation invariant. Post-hoc calibration produces

a further,

independent

failure

mode

in which isotonic

regression destroys cross-chemistry discriminative power

under distribution shift.

The

embedded deployment

adds

a concrete

positive

result to these cautions. Three ensembles totaling 900 trees

and 26,336 nodes fit into 372 kB, run on a $12 microcon-

troller in about 2.7 ms for all three models from PSRAM,

about 0.5 ms for a single model

kept in internal

SRAM,

and reproduce

the

Python training libraries

to within

−6

1.1 × 10

across more than a thousand validation rows.

Future work should pursue learned feature representations

built for chemistry invariance,

bidirectional

transfer,

and

multi-seed GRU evaluation.

Fig.

12.

Deployment trade-offs:

(a) measured single-row inference

time per model at 240 MHz (PC, and ESP32-S3 with the binary in

flash,

PSRAM,

or a single model

in internal

SRAM);

(b) memory

Acknowledgment

footprint of the 372 kB binary as a share of each tier, exceeding the

ESP32-S3’s roughly 320 kB of free internal SRAM.

This

work

received

no

external

funding.

The

authors

declare

no

conflicts

of

interest.

The

source

code,

data,

trained

models,

firmware,

and

validation

pipeline

are

publicly

available

at

https://github.com/touhidsiddiqueeraj-bit/

Multi-Horizon-Hazard-Models-for-Battery-Failure-Prediction.

References

[1]

T.

A.

Shikdar and H.

Laaksonen,

“Learning when not to use

a battery: Multihorizon failure intelligence,” Int. Trans. Electr.

Energy Syst., vol. 36, Art. no. 6000810, 2026.

[2]

H.

Meng and Y.

Li,

“A review on prognostics

and health

management (PHM) methods of lithium-ion batteries,” Renew.

Sustain. Energy Rev., vol. 116, Art. no. 109405, 2019.

[3]

L. Breiman, “Random forests,” Mach. Learn., vol. 45, no. 1, pp.

5–32, 2001.

[4]

T. Chen and C. Guestrin, “XGBoost: A scalable tree boosting

system,” in Proc. ACM SIGKDD, 2016, pp. 785–794.

[5]

G. Ke, Q. Meng, T. Finley, et al., “LightGBM: A highly eﬀicient

gradient boosting decision tree,” in Proc. NeurIPS, vol. 30, 2017,

Fig.

13.

On-device

C engine

predictions

versus

Python pre-

pp. 3146–3154.

dict_proba() over all 1,028 rows and three models (agreement scatter

[6]

B.

Saha and K.

Goebel,

“Battery data set,” NASA Ames

on the identity line,

inset residual

histogram of C minus Python in

Prognostics Data Repository, 2007.

−6

−7

units of 10

). Maximum absolute errors are 2.4 × 10

(XGBoost),

[7]

CALCE Battery Research Group,

“Battery aging datasets,”

−6

−8

1.1 × 10

(LightGBM), and 3.0 × 10

(Random Forest).

Univ. Maryland, 2023.

[8]

C.

R.

Birkl

and D.

A.

Howey,

“Oxford battery degradation

dataset 1,” Univ. Oxford, 2017.

Severson set is included specifically to offset this and it

[9]

K.

A.

Severson,

P.

M.

Attia,

N.

Jin,

et

al.,

“Data-driven

prediction of

battery cycle life before capacity degradation,”

reproduces

every qualitative finding.

The analysis

runs

Nature Energy, vol. 4, pp. 383–391, 2019.

one direction,

LCO to LFP,

and bidirectional

transfer

[10]

S. Sahoo, K. S. Hariharan, S. Agarwal, et al., “Transfer learning

would round out the picture. The GRU is evaluated with

based generalized framework for state of

health estimation of

Li-ion cells,” Sci. Rep., vol. 12, Art. no. 13173, 2022.

a single seed; the instability across configurations is itself

[11]

J. Lu, R. Xiong, J. Tian, C. Wang, and F. Sun, “Deep learning to

informative, but multi-seed analysis is left to future work.

estimate lithium-ion battery state of health without additional

degradation experiments,” Nat.

Commun.,

vol.

14,

Art.

no.

2760, 2023.

VI.

Conclusion

[12]

J. C. Platt, “Probabilistic outputs for support vector machines

This study widens a multi-horizon hazard classification

and comparisons to regularized likelihood methods,” in Adv.

Large Margin Classifiers, MIT Press, 1999, pp. 61–74.

framework,

first

shown on one model

and one dataset,

[13]

B.

Zadrozny and C.

Elkan,

“Transforming classifier

scores

to four model

families on two LCO datasets,

with cross-

into accurate multiclass probability estimates,” in Proc.

ACM

chemistry transfer checked on two independent LFP tar-

SIGKDD, 2002, pp. 694–699.

MULTI-HORIZON HAZARD MODELS FOR BATTERY FAILURE PREDICTION

11

[14]

A.

Niculescu-Mizil

and R.

Caruana,

“Predicting good proba-

bilities

with supervised learning,” in Proc.

ICML,

2005,

pp.

625–632.

[15]

L. Huang, J. Zhao, B. Zhu, H. Chen, and S. K. L. M. v. Broucke,

“An experimental

investigation of

calibration techniques

for

imbalanced data,” IEEE Access,

vol.

8,

pp.

127343–127352,

2020.

[16]

S.

M.

Lundberg,

G.

G.

Erion,

H.

Chen,

et

al.,

“From local

explanations to global

understanding with explainable AI for

trees,” Nat. Mach. Intell., vol. 2, no. 1, pp. 56–67, 2020.

[17]

P.

Grzesik and D.

Mrozek,

“Combining machine learning and

edge computing:

Opportunities,

challenges,

platforms,

frame-

works, and use cases,” Electronics, vol. 13, no. 3, Art. no. 640,

2024.

[18]

C. Gupta and A. Ramdas, “Online Platt scaling with calibeat-

ing,” in Proc. ICML, PMLR 202, 2023, pp. 12182–12204.

[19]

E.

R.

DeLong,

D.

M.

DeLong,

and D.

L.

Clarke-Pearson,

“Comparing the areas under two or more correlated receiver

operating characteristic curves:

A nonparametric approach,”

Biometrics, vol. 44, no. 3, pp. 837–845, 1988.